Malaysian Journal of Analytical Sciences Vol 20 No 6 (2016): 1373 - 1381

EFFECT OF HEAT TREATMENTS ON THE YIELD, QUALITY AND STORAGE STABILITY OF OIL EXTRACTED FROM PALM FRUITS

# (Kesan Rawatan Haba Terhadap Peratus Pengekstrakan Minyak, Kualiti dan Kestabilan Semasa Penyimpanan Minyak yang Diekstrak Daripada Buah Kelapa Sawit)

# Hasimah Kasmin1, 2, Azwan Mat Lazim2, Roila Awang1*

1Engineering & Processing Research Division,

Malaysian Palm Oil Board, No. 6, Persiaran Institusi, Bandar Baru Bangi, 43000 Kajang, Selangor, Malaysia

# 2School of Chemical Sciences and Food Technology, Faculty of Science and Technology,

# Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

# *Corresponding author: roila@mpob.gov.my

Received: 17 August 2015; Accepted: 29 August 2016

Abstract

This paper presents the impact of heat treatments on the yield and quality of the palm oil using solvent extraction techniques. Heating temperature range from 70 to 90 °C and heating time range from 30 to 90 min were studied. The quality parameters measured were free fatty acids (FFA), deterioration of bleaching index (DOBI), carotene content, oxidative stability and fatty acid composition of the oil. These parameters were used to determine CPO quality which has been stored for 3 months. It was found that the yield significantly increased with the increasing of heating temperature and time. The optimum parameter was achieved with heating temperature of 90 °C and heating time of 60 min via wet heating. Optimum oil yield obtained was 24% with 0.93% FFA content, 6.10 DOBI, 938.42 ppm carotene content and oxidative stability of 26.87 hours. Fatty acid composition (FAC) of oil extracted showed a reduction in unsaturated fatty acids as the heating duration increased. After 3 months of storage, 60 min of heating time via wet heating process was recorded the lowest increment of FFA (1.32%) and low reduction of DOBI value (0.41). However, carotene content gave the highest loss with 113.54 ppm towards the end of the storage.

Keywords:  crude palm oil, heat treatments, oil yield, oil quality, storage stability

Abstrak

Kajian ini bertujuan untuk menganalisis kesan rawatan haba semasa pensterilan ke atas peratus pengekstrakan minyak dan kualiti minyak sawit mentah (MSM) dengan menggunakan pelarut bagi mengekstrak MSM. Suhu dan masa pensterilan yang berlainan telah digunakan bagi mengekstrak MSM iaitu suhu pensterilan dari 70 hingga 90 °C manakala masa pensterilan dari 30 hingga 90 min. Kesan rawatan haba ke atas peratus pengekstrakan minyak telah dijalankan. Kualiti MSM yang diekstrak iaitu asid lemak bebas (ALB), indeks pemerosotan terhadap pelunturan (DOBI), kandungan karotena, kestabilan oksidatif dan komposisi asid lemak yang telah dicirikan. Parameter ini digunakan untuk menentukan kualiti MSM yang telah disimpan selama 3 bulan. Hasil kajian mendapati peratus pengekstrakan minyak meningkat secara signifikan dengan kenaikkan kesan suhu dan masa.  Hasil optimum pada tahap maksimum iaitu sebanyak 24% telah dicapai pada suhu pensterilan 90 °C dan masa pensterilan 60 min dengan menggunakan pemanasan basah. Kualiti minyak pula menunjukkan kandungan ALB sebanyak 0.93%, DOBI sebanyak 6.10, kandungan karotena sebanyak 938.42 ppm dan kestabilan oksidatif selama 26.87 jam. Selepas 3 bulan penyimpanan, masa pemanasan selama 60 min menggunakan pemanasan basah telah menunjukkan peningkatan kandungan ALB dan penurunan bacaan DOBI yang paling rendah iaitu masing – masing 1.32% dan 0.41. Namun, kandungan karotena didapati merosot paling tinggi iaitu sebanyak 113.54 ppm.

Kata kunci: 	minyak sawit mentah, rawatan haba, peratus pengekstrakan minyak, kualiti minyak, kestabilan minyak

References

- Mahidin, M. R. (1998). Quality improvement in the production of Malaysian palm oil. Palm Oil Development, 9: 15 - 21.

- Chow, M. C. and Ma, A. N. (2007). Processing of fresh palm fruits using microwaves. Journal of Microwave Power & Electromagnetic Energy, 40 (3): 165 - 173.

- Ariffin, A. A. (1991). Chemical changes during sterilization process affecting strippability and oil quality. In Seminar on Developments in Palm Oil Milling Technology and Environmental Management: pp. 2 - 12.

- PORIM (1991). International Palm Oil Conference Chemistry & Technology. pp. 269.

- Cheng, S. F., Mohd Nor, L. and Chuah, C. H. (2011). Microwave pretreatment: A clean and dry method for palm oil production. Journal of Industrial Crops and Products, 34: 967 - 971.

- Nu’man, A. H., Ng, M. H., Choo, Y. M. and Ma, A. N. (2012). Dry heating of palm fruits: Effect on selected parameters. American Journal of Engineering and Applied Sciences, 5(2): 128 - 131.

- Mohd Fauzi, N. A., Sarmidi, M. R. and Chua, L. S. (2011). Metabolites profiling of heat treated palm oil extract. Journal of Applied Sciences, 11(13): 2376 - 2381.

- Mohd Fauzi, N. A. and Sarmidi, M. R. (2011). Extraction of heat treated palm oil and their stability on β-carotene during storage. Journal of Science and Technology, 2(1): 45 - 54.

- Malaysian Palm Oil Board (2004). MPOB Test Methods. Malaysian Palm Oil Board, Ministry of Primary Industries, Malaysia.

- Baryeh, E. A. (2001).  Effect of palm oil processing parameters on yield. Journal of Food Engineering, 48(1): 1- 6.

- Malaysian Palm Oil Board (2010). Malaysian Oil Palm Statistic 2009. Kajang. Malaysia.

- Sambanthamurthi, R., Sundram, K. and Tan, Y. A. (2000). Chemistry and biochemistry of palm oil. Progress in Lipid Research, 39: 507 - 558.

- Macaire, W. H., Bernard, T., Noel, T., Felicite, T. M., Cesar, K., Michel, L. and Jacques, F. (2010). Extraction of palm kernel oil in Cameroon: effects of kernels drying on the oil quality. Journal of Food Technology, 8(1): 1 - 7.

- Owolarafe, O. K. and Faborode, M. O. (2007). Micro-structural characterisation of palm fruit at sterilisation and digestion stages in relation to oil expression. Journal of Food Engineering, 85: 598 –605.

- Yung, C. L., Choo, Y. M., Cheng, S. F., Ma, A. N, Chuah, C. H. and Yusof, B. (2006). The effect of natural and synthetic antioxidants on the oxidative stability of palm diesel. Fuel, 85: 867 - 870.

- Noraini, S. and Siew, W. L. (1990). Quality control measures in palm oil industry. Palm Oil Dev.

- Vijaya, S., Choo, Y. M., Muhammad, H., Hasyim, Z. and Yew, A. T. (2010). Life cycle assessment of the production of crude palm oil (part 3). Journal Oil Palm Research, 22: 895 - 903.

- Chong, C. L. (1994). Chemical and physical properties of palm oil and palm kernel. Selected readings on palm oil and its uses. Malaysia: Palm Oil Research institute of Malaysia: pp. 60 - 67.